Circuit Documentation

Summary

This circuit is designed to interface an ESP32 microcontroller with an HC-SR04 ultrasonic sensor, an OLED display, an active buzzer, and a couple of resistors. The ESP32 serves as the central processing unit, controlling the ultrasonic sensor for distance measurement, displaying information on the OLED, and activating the buzzer based on certain conditions. The resistors are used for signal conditioning.

Component List

- ESP32 38p Type-C CP2102

- Description: A versatile microcontroller with Wi-Fi and Bluetooth capabilities.

- Pins: 5V, CMD, SD3, SD2, G13, GND, G12, G14, G27, G26, G25, G23, G32, G35, G34, SN, SP, EN, 3V3, CLK, SD0, SD1, G15, G2, G0, G4, G16, G17, G5, G18, G19, G21, RXD, TXD, G22

- HC-SR04 Ultrasonic Sensor

- Description: A sensor used for measuring distance by using ultrasonic waves.

- Pins: VCC, TRIG, ECHO, GND

- Resistor (1k Ohms)

- Description: A resistor used for signal conditioning.

- Pins: pin1, pin2

- Properties: Resistance: 1000 Ohms

- Resistor (2k Ohms)

- Description: Another resistor used for signal conditioning.

- Pins: pin1, pin2

- Properties: Resistance: 2000 Ohms

- Active Buzzer

- Description: A buzzer that produces sound when activated.

- Pins: Positive, Negative

- OLED Display

- Description: A display module used for visual output.

- Pins: GND, VCC, D0, D1, RES, DC, CS

Wiring Details

ESP32 38p Type-C CP2102

- 5V is connected to VCC of the HC-SR04 Ultrasonic Sensor.

- GND is connected to:

- Negative of the Active Buzzer

- GND of the HC-SR04 Ultrasonic Sensor

- GND of the OLED Display

- pin1 of the 2k Ohm Resistor

- G18 is connected to pin2 of both the 1k Ohm and 2k Ohm Resistors.

- G5 is connected to TRIG of the HC-SR04 Ultrasonic Sensor.

- G23 is connected to Positive of the Active Buzzer.

- 3V3 is connected to VCC of the OLED Display.

- G4 is connected to RES of the OLED Display.

- G2 is connected to DC of the OLED Display.

- G14 is connected to D0 of the OLED Display.

- G13 is connected to D1 of the OLED Display.

- G15 is connected to CS of the OLED Display.

HC-SR04 Ultrasonic Sensor

- VCC is connected to 5V of the ESP32.

- TRIG is connected to G5 of the ESP32.

- ECHO is connected to pin1 of the 1k Ohm Resistor.

- GND is connected to GND of the ESP32.

Resistor (1k Ohms)

- pin1 is connected to ECHO of the HC-SR04 Ultrasonic Sensor.

- pin2 is connected to G18 of the ESP32.

Resistor (2k Ohms)

- pin1 is connected to GND of the ESP32.

- pin2 is connected to G18 of the ESP32.

Active Buzzer

- Positive is connected to G23 of the ESP32.

- Negative is connected to GND of the ESP32.

OLED Display

- GND is connected to GND of the ESP32.

- VCC is connected to 3V3 of the ESP32.

- D0 is connected to G14 of the ESP32.

- D1 is connected to G13 of the ESP32.

- RES is connected to G4 of the ESP32.

- DC is connected to G2 of the ESP32.

- CS is connected to G15 of the ESP32.